See discussions, stats, and author profiles for this publication at: https://www.researchgate.net/publication/255700594

Estimation of the sea state bias in radar altimeter measurements of sea level: Results from a new nonparametric method

Article  in  Journal of Geophysical Research Atmospheres · July 1998

DOI: 10.1029/98JC01194

CITATIONS

76

2 authors, including:

Philippe Gaspar

Mercator Ocean

122 PUBLICATIONS   4,466 CITATIONS

SEE PROFILE

Some of the authors of this publication are also working on these related projects: INDESO Mangrove Project View project

SEAPODYM View project

All content following this page was uploaded by Philippe Gaspar on 28 May 2014.

The user has requested enhancement of the downloaded file.

READS 214

JOURNAL OF GEOPHYSICAL RESEARCH, VOL. 103, NO. C8, PAGES 15,803-15,814, JULY 15, 1998

Estimation of the sea state bias in radar altimeter measurements  of sea level: Results from a new nonparametric method

Philippe Gaspar

Collecte Localisation Satellites, Space Oceanography Division, Ramonville, France

Jean-Pierre Florens

Groupe de Recherche en Economie Mathematique et Quantitative-Institut d'Economie Industrielle  Universit6 des Sciences Sociales, Toulouse, France

Abstract. The sea state bias (SSB) affecting altimetric measurements of the sea surface  height (SSH) is classically estimated using empirical parametric models. The model  parameters are determined to minimize the variance of the SSH differences at crossover  points or along collinear tracks. It is shown here that so-calibrated models are not true  least squares approximations of the SSB. Simulations indicate that the difference from a  true least squares solution is typically a few millimeters to a few centimeters, varying with  sea state. This difference proves to be the result of the inevitably imperfect specification  of the model's parametric form, which corrupts the calibration process when performed on  SSH differences rather than directly on SSH measurements. To avoid specification of a  parametric form, we present a new nonparametric version of the SSB estimation problem  and propose a general solution based on the statistical technique of kernel smoothing.  This solution is then used to obtain the first fully nonparametric estimate of the TOPEX  altimeter SSB as a function of both the wind speed and the wave height. The obtained  estimate, though based on a limited data set, proves to be better than those obtained with  the standard parametric SSB models featured in the TOPEX-POSEIDON Geophysical  Data Records. The improvement is most significant in middle- and high-latitude oceans.

1. Introduction

Yaplee et al. [1971] first observed that the sea surface radar  cross section per unit area is larger toward the wave troughs  and smaller toward the crests. As a result, the sea level mea sured by radar altimeters appears to be lower than the true  mean sea level. This effect is known as the electromagnetic  bias. Technical estimation problems also give rise to two other  sea-state-related biases: the skewness and tracker biases (for  details, see Chelton et al. [1989]). These are generally lumped  together with the electromagnetic bias to form the so-called  sea state bias (SSB). The SSB is typically a few percent of the  significant wave height (SWH) and thus ranges between a few  centimeters and a few decimeters.

With a standard deviation as large as 2 cm, the SSB estima tion error is, today, the largest error affecting the range mea surements [Fu et al., 1994] of the TOPEX and POSEIDON  altimeters. The relatively poor quality of the SSB estimation is

largely due to our limited understanding of the mechanisms  that generate the SSB, and the electromagnetic bias, in partic ular. In the absence of well-established theoretical guidelines,  a wide variety of empirical models have been developed, nat urally leading to some controversy on the choice of the most  appropriate one [e.g., Rodriguez and Martin, 1994; Glazman et  al., 1994]. Still, all empirical models rely on the hypothesis that  the SSB can be modeled under the general form

SS• = SWI-I b(x, 0) (•)

Copyright 1998 by the American Geophysical Union.

Paper number 98JC01194.

0148-0227/98/98JC-01194509.00

where b is a specified function of x, a vector of pertinent  sea-state-related variables, and 0, a vector of parameters. The  function b is sometimes referred to as the relative bias. In  practice, the components of x are systematically chosen among  the few sea-state-related variables that can be directly mea sured by an altimeter: SWH, the wind speed (U), the back scatter coefficient (fro), or any combination thereof. Most mod els actually assume that b is a simple polynomial function of U  and/or SWH [e.g., Born et al., 1982; Douglas and Agreen, 1983;  Zlotnicki et al., 1989; Melville et al., 1991; Ray and Koblinsky,  1991; Gaspar et al., 1994; Chelton, 1994], but other parametric  forms have been used [Fu and Glazman, 1991; Glazman et al.,  1994]. In all cases, the model parameters are determined to  minimize the variance of the measured sea surface height  (SSH) differences at crossover points or along collinear tracks.  We show in this paper (section 2) that the set of model pa rameters that minimizes the variance of the crossover or col linear differences is not the set of model parameters that min imizes the distance between the true SSB and the model  approximation. The obtained model is thus not a true least  squares approximation of the SSB. It differs from it by, typi cally, a few millimeters to a few centimeters. This problem  proves to be a consequence of the inevitably imperfect speci fication of the model's parametric form. An incomplete or not  fully adequate model formulation generates autocorrelated er rors which affect the model calibration obtained with SSH  differences rather than directly with SSH measurements.

The search for a solution to this problem motivated the  present work. Having recognized that the origin of the problem  lies in the parametric approach, one is naturally led to consider  use of nonparametric estimation methods. These methods

15,803

15,804 GASPAR AND FLORENS: NONPARAMETRIC ESTIMATION OF SEA STATE BIAS

eliminate the rigidity of parametric methods, and the resulting  errors, by removing the restriction that the fitted model be longs to a parametric family. Another advantage of these  methods is that they leave no place for controversy about the  choice of the functional form of the model. This choice is  simply not needed.

Even if the above mentioned problem of the parametric  approach had not been previously identified, a few authors had  recognized that the use of parametric models might overly  constrain the simulated behavior of the SSB and had devel

oped nonparametric solutions. Witter and Chelton [1991a] first  obtained a simple nonparametric estimation of the Geosat  relative bias assuming that b was an unspecified function of  either U or SWH. This function was estimated piecewise over  discrete U or SWH intervals. The original implementation of  the method proved to be incorrect but was subsequently cor

rected by Chelton [1994] and applied to TOPEX data. Ro driguez and Martin [1994] also estimated the TOPEX SSB in a  partly nonparametric way. They assumed that the relative bias  could be modeled as the sum of a nonparametric function of U  and a linear function of SWH. The function of U was approx imated by a sum of local basis functions. These models thus  allow nonparametric variations of the SSB in only one dimen

sion (U or SWH) and still conform to the usual formulation  (equation (1)). Limitation to the one-dimensional case is more  guided by numerical constraints than by the physics of the  problem, and use of (1) appears to be a convenient, but not  indispensable, choice. Indeed, it is now clear that the SSB  dependence on SWH is not strictly linear. The results of Witter  and Chelton [1991a], Gaspar et al. [1994], and Chelton [1994]  show that the magnitude of the relative bias decreases when  SWH increases. Theoretical models also suggesthat the SSB  dependence on SWH is weaker than linear [Barrick and Lipa,  1985; Fu and Glazman, 1991]. There is thus no compelling  reason to formally single out SWH in the specification of a SSB  model.

The SSB estimation method developed in this paper is mul tivariate and fully nonparametric. It avoids use of (1) and  simply assumes that SSB is a (nonspecified) function of a set of  pertinent variables. The SSB estimation problem is reformu lated in section 3 using the now-classical concept of nonpara metric regression. A general solution based on the statistical  technique of kernel smoothing is presented. This solution is  then used to estimate the SSB of the TOPEX Ku-band altim eter, based on a data set described in section 4. Section 5 deals  with the numerical implementation of the solution. The results  are presented in section 6 and compared with those obtained  with commonly used parametric models.

This paper focuses on the practical implementation of the  new estimation technique and on the discussion of the results  obtained for the TOPEX SSB. A more theoretical analysis of  the nonparametric SSB estimation problem and of the statis tical properties of the obtained solution is given by J.-P. Flo rens and P. Gaspar (manuscript in preparation, 1998).

2. On the Use of Parametric Models

for SSB Estimation

2.1. Basis for Parametric SSB Estimation

Let us denote h' a the altimetric range measurement not  corrected for the sea state bias. The difference between the  satellite altitude above the reference ellipsoid and h • yields a

biased sea surface height measurement (SSH') that contains  the geoid signal (he), the ocean dynamic topography (r/), the  SSB, and some measurement noise (w):

SSH' = hg + r/+ SSB + w (2)

The geoid signal, largely dominant but still imperfectly known,  is easily eliminated from this equation by forming differences  between two SSH' measurements taken at different times but  at the same geographic location (either along collinear tracks  or at crossover points). Noting these two measurements SSHi  and SSH•, one obtains

SSH•- SSH• = (SSB2- SSB 0 + (T•2--T•i) -t- (W 2 --W1) (3)

In the context of $SB estimation, the change in dynamic to pography (r/2 - r/l) is considered as noise and lumped together  with (w2 - w•) to form a single zero-mean noise term e:

SSH•- SSH• = (SSB2- SSB•) + e (4)

This observation equation is central to all $$B estimation tech niques that rely on altimeter data only. On the basis of this  equation, estimation of $$B is simple if a parametric form of  the bias is assumed. The commonly used formulation is (1), but  a more general expression is

SSB: 0)

where 92 is any specified function of the pertinent variables x  and the parameters 0. Once the functional form of 92 is chosen,  sets of altimetric measurements can be used to determine the  value of 0 that minimizes the variance of the sea surface height  (SSH) differences after correction for the (modeled) SSB. This  variance reads

E[SSH2 -- SSH•] 2= E[(SSH•- 922) -- (SSH•- 92•)]2 (6)

where E denotes an expectation and the abbreviated notation  92i is used for 92(xi, 0). In most published models, 92 is a linear  function of 0 so that the minimization problem reduces to a  simple, generally multivariate, regression problem [e.g., Ray  and Koblinsky, 1991]. Standard minimization techniques also  exist to deal with nonlinear models [e.g., Fu and Glazman,  1991].

A word of caution must be added about the use of the  parametric form (5) instead of the more usual form (1). In deed, the variance to minimize (6) only contains $$B differ ences. Therefore adding a constant to any $$B estimate that

minimizes (6) will generate another minimum variance solu tion. To eliminate this undetermination, one particular value  of the $$B must be specified. This must be explicitly done when  using (5). It is implicitly done when using (1), as this particular

model formulation imposes $$B = 0 when $WH = 0.

2.2. Properties of the Parametric SSB Estimates  Using (4), the variance of the SSH differences can be re written

E[SSH2- SSH,] 2= E[(SSB2- 922) -- (SSB,- 92•) + 812 (7)

Assuming, as usual, that e and SSB are uncorrelated, (7) be comes

E[SSH2- SSH•] 2= 2E[SSB- 92]2

- 2E[(SSB•- 921)(SSB 2 --922)] q- E[e] 2 (8)

GASPAR AND FLORENS: NONPARAMETRIC ESTIMATION OF SEA STATE BIAS 15,805

where the variance of the model error E[SSB - •]2 = E[SSB•  _ %]2 = E[SSB2 _ •212. Equation (8) clearly shows that the  value of 0 that minimizes E[ SSH 2 - SSH•] 2 actually minimizes

E[SSB- ½]2_ E[(SSB1- ½•)(SSB2- ½2)] (9)

In general, this value of 0 does not realize the least squares  approximation of SSB by ½ since it does not minimize  E[SSB - ½]2. A special case where the same 0 minimizes both  (9) and E[SSB - ½]2 is the case of a perfectly specified model,  i.e., a model for which there exists a value of 0, say, 0o, such  that ½(x, 0o) is the true SSB. In that case, both E[SSB - ½]2  and (9) are minimized (equal to zero) for 0 = 0o. This shows  that the use of measurement differences allows correct cali bration of a perfect SSB model. One can hardly claim, how

tween the two SSB estimates range from a few millimeters at  low wave heights regularly increasing up to 3 cm for wave  heights close to t0 m. Differences also vary slightly with wind  speed. A similar pair of regressions was performed, this time  with the most simple one-parameter model (BMt):

BMi(SWH) -- aSWH (•3)

In this case, the difference between the two model calibrations  is equal to 0.4% of SWH, a 20% error for a typical SSB of  -2% of SWH. It results in a 4-cm difference between the two  SSB estimates for SWH - t0 m. This simple exercise thus

shows that usual SSB models calibrated on crossover differ ences can be centimeters off the true least squares solution. In  addition, this difference from the least squares solution varies

ever, that parametric SSB models are perfect. Therefore the  autocorrelation of the model error, E[(SSB• - ½i)(SSB 2 -  ½2)], affects the SSB estimation process and causes the SSB  estimate to deviate from the true least squares solution. Is the  deviation significant? This cannot be told from the altimeter  data themselves, as the very large geoid signal in SSH' mea surements prevents any precise determination of the true least  squares approximation. Still, a simple simulation can provide  useful insights.

2.3. Evaluation of the Deviation From the True  Least Squares Solution

The goal of the simulation is to obtain a reasonable estimate  of the difference between a true least squares estimate of the  SSB and the more usual solution obtained by fitting a para metric model on measurement differences. To avoid estima tion accuracy problems, we generated a set of "idealized SSH'  measurements" containing only a known SSB plus some noise,  but no geoid or dynamic topography signal. To obtain this, we  took a set of about 130,000 TOPEX altimeter measurements at  crossover points, representing the 20 first repeat cycles of the  TOPEX data set completely described in section 4. We then  replaced the SSH' data field by a new field (z) made of a SSB  simulated by the BM4 model [Gaspar et al., 1994] plus a Gauss ian random noise:

z = BM4(U, SWH) + w (t0)

where

BM4(U, SWH)= SWH[al + a2U + a3U 2 + a4 SWH] (11)

The values of the a i parameters are those recommended by  AV!SO [1996] for TOPEX data. The standard deviation of the  noise (w) is 6.3 cm, so that the standard deviation of the  crossover differences after SSB correction (w 2 - w•) is  6.3X/• • 9 cm, a realistic value for data of the TOPEX class.

On the basis of this data set, we proceeded with the calibra tion of an "imperfect" SSB model, in this case the widely used  three-parameter model (BM3) of Hevizi et al. [1993]:

BM3(U, SWH) = SWH[a• + a2U q- a3 U2] (12)

BM3 can obviously not provide a perfect fit to data generated  with the more complete BM4 model. A first regression was  performed using the z data themselves to obtain the true least  squares approximation of BM4 by a BM3 model. A second  regression was then performed using the crossover differences  (z2 - z•). As expected, the two regressions yield different  parameter estimates for BM3. The resulting differences be

with sea state. This can induce spurious SSH gradients between  ocean regions with different sea surface conditions. Of course,  this error is only a fraction of the total SSB estimation error,  but it is a nagging one, as it is built in the parametric approach:

The unavoidably imperfect specification of the model's para metric form corrupts its calibration when performed on SSH'  differences rather than directly on SSH'.

One way out of this problem is to avoid the specification of  a parametric form, which naturally leads us to consider non parametric estimation techniques. In the next section we re formulate the SSB estimation problem as a special nonpara metric regression problem. It is special because the regression  only uses measurement differences, not the measurements  themselves. An original solution is presented, based on the  now well-developed kernel smoothing technique.

3. Nonparametric SSB Estimation

3.1. Basis for Nonparametric Regression

Let us briefly recall the theoretical basis of nonparametric  regression. Assume we have a random scalar variable • jointly  distributed with a random vector x. Given a set of n observa tions (•i, x/), we want to estimate the regression function r(x)  - E[•lx] without making assumptions concerning the func tional form of r. Different technical solutions exist [e.g.,  Hi•rdle, 1990]. Here we use the so-called kernel smoothing  method that has the advantage of being quite intuitive and  relatively simple to implement. Since the seminal papers of  Rosenblatt [1956] and Parzen [t962], this technique has been  extensively studied. It has now a strong theoretical basis [see,  e.g., Wand and Jones, 1995].

The kernel estimator f(x) of the conditional expectation  E[•lx ] is simply a weighted mean of the observed •i, the  weights being a decreasing function of a distance between x  and xi:

?(x) = • •ia,(x- xi) (14)

where % is a weighting function whose general form reads  •( X h• --

O/n(X- Xi)-- (15)

i=1 x -- X i

K is a kernel function, h. is the bandwidth vector, and the  index n indicates that the bandwidth depends on the number of

15,806 GASPAR AND FLORENS' NONPARAMETRIC ESTIMATION OF SEA STATE BIAS

measurements. A kernel is a symmetric scalar function satis fying • K(x) dx: 1. It is usually chosen to be a probability  density function. The bandwidth vector has the same size as x  and is made of positive numbers. The ratio [x - xi/hn] is simply  defined component by component.

3.2. The SSB Case

Let us now concentrate on the specific SSB estimation prob lem. Simply start with the assumption that SSB is a (nonspeci fied) function ½ of x, a vector of p pertinent variables. As  previously indicated, we will actually use x - (U, SWH), but  this does not need to be specified at this stage. Noting y -  SSH} - SSHi, equation (4) can be rewritten

I - A. We now have a system of n equations with n - 1  unknowns. A linear least squares solution is readily obtained:

½ = (g•gl)-lg•(Ay- g0½0) (23)

This provides the missing estimates of •(xu), i -- 2, n. These  can then be plugged into (18) to estimate •(x) for any value of  x. The problem is thus formally solved. Provided that the  bandwidth is properly chosen, it can be shown (J.P. Florens  and P. Gaspar, manuscript in preparation, 1998) that the SSB  estimator (equation (23)) is consistent and asymptotically nor mal. It is thus exempt from the problems affecting imperfectly  specified parametric models.

The price to pay for the use of this nonparametric approach  is the inversion of a [(n - 1) x (n - 1)] matrix, as required

y: (x0- (x0 +

which implies

E[ylx- x]: - E[(x01x: x]

(16)  (17)

by (23). This becomes rapidly intractable as the number of  measurements increases. We demonstrate in section 6 that  useful solutions can be obtained with a reasonably small  amount of data. Another potential problem is that there is no

Having at our disposal a set of n measurements (Yi, Xli, x2i),  the kernel estimator (14) can be used to evaluate the condi tional expectations in (17). One obtains

= +

t--1 t--1

This relation provides a practical means of estimating •(x), for  any x, provided that the values of •(xu) are known. In other  words, the problem of estimating •(x) is now reduced to the  problem of estimating • for a finite set of values of x. Equation  (18) being valid for any of these values of x, one can write

92(x•j) -- • yian(x•j- xzi) + • •(Xli)O•n(Xlj- X2i) (19)  t=l t=l

Vj=l,...,n

or, in matrix form,

(I - A)q•, = Ay (20)

where I is the (n x n) identity matrix, A is the (n x n) matrix  with elements aii = an(Xlj- x2i), qh r = [•(xn), '",  •(Xln)], and yr = [y•, ..., y,,].

Unfortunately, the linear system (20) cannot be solved for  q• since I - A is singular. Indeed, the definition of the weights  (equation (15)) implies that the sum of elements, in each line  of this matrix, is zero. The rank of I - A is thus equal to n -  1. We find here the numerical expression of the undetermina tion problem mentioned at the end of section 2.1. To fix it, we  have to impose (rather than estimate) the value of one of the  components of q•, say, •(xll ), as the data set can always be  rearranged to have the selected component in first place. The  constraint is thus

½(x,,) = ½0 (21)

The choice of an appropriate X•l and ½o will be discussed in  section 5.3. Given (21), equation (20) can be rewritten

Blq• = Ay- B0•0 (22)

where q• is the vector made of the n - 1 components of q•l that  remain to be determined: q•r = [•(x12), ... ' •(Xln) ] and (Bo,  B1) is a partition of I - A, B o being simply the first column of

guarantee that B•rB1 is not numerically singular or nearly so.  We have not encountered this situation in the numerical ex periments presented in section 6, but should this happen, the  way to solve the problem is to impose the value of more than  one of the components of q•. The formal solution (equation  (23)) is still valid but with a different (Bo, B1) partition of the  I - A matrix. If m components of q•l have to take specified  values, Bo is made of the m first columns of I - A.

4. Data Processing

To evaluate the SSB estimation technique presented in sec tion 3, we used it to estimate the SSB of the TOPEX Ku-band  altimeter. This estimation is based on 100 cycles of TOPEX  measurements obtained between April 4, 1993, and April 15,  1996. This period actually covers 111 ten-day repeat cycles  (cycles 21-131) from which we have discarded 10 POSEIDON  cycles and a missing TOPEX cycle (cycle 118). Use is made of  the crossover data sets provided byAVISO [1996] based on the  recently reprocessed TOPEX-POSEIDON Geophysical Data  Records (GDRs). The crossovers are computed cycle per cy cle. The crossover differences (y - SSH• - SSH'•) correspond  to the descending arc measurement minus the ascending arc  measurement. Therefore index 1 (index 2) denotes measure ments taken on the ascending arc (descending arc) in the rest  of this paper.

SSH' is computed using the CNES orbit and the TOPEX  Ku-band range measurements to which all standard correc tions except the SSB are applied. More precisely, we apply the  dry tropospheric and inverse barometer corrections deduced  from the European Centre for Medium-Range Weather Fore casts (ECMWF) pressure analyses, the wet troposphericor rection deduced from TOPEX microwave radiometer mea surements, the ionospheric correction deduced from the dual frequency TOPEX altimeter measurements, the ocean tide  computed with the Center for Space Research (CSR) 3.0  model, the solid Earth, and pole tides. The wind speed (U) is  the 10-m wind speed computed by AVISO [1996] using the  Ku-band backscatter coefficient (fro) and the modified Chel ton-Wentz algorithm [Witter and Chelton, 1991b].

Stringent editing is applied to the data. All measurements  with missing or unreliable corrections are eliminated. Also  discarded are the measurements with rro < 7 dB, o' o > 30 dB,  or SWH > 12 m. After quality control a total of nearly 633,000  valid crossover differences are obtained.

GASPAR AND FLORENS: NONPARAMETRIC ESTIMATION OF SEA STATE BIAS 15,807

5. Numerical Implementation

5.1. Choice of the Kernel Function and Bandwidth Vector

Estimation of the SSB based on (18) and (23) requires the

specification of a kernel and the selection of an appropriate  bandwidth vector. There is an infinite number of kernel func tions, but, in practice, only a few are commonly used [see, e.g.,  Wand and Jones, 1995]. These have very similar statistical prop erties, and a specific choice is generally motivated by a specific  requirement (such as, for example, the need to obtain estima tots with continuous first derivatives). Such requirements have  not (yet) been identified in the SSB estimation problem. We  will therefore make the very common and conservative choice  of a Gaussian kernel. For x = (U, SWH) this kernel takes the  form

= exp K h n 2 rrh vh SWH 2h •

ß exp [-(SWH-SWH,) 2h: 2] (24) SWH

Choice of the bandwidth is more delicate. The bandwidth is  clearly a smoothing parameter: A larger bandwidth yields a  smoother estimate. Of course, the goal is not to overly smooth  the solution but to select a bandwidth that minimizes a chosen  measure of the distance between SSB and its estimate •. Op timal bandwidth selection is generally an iterative process. An  initial value of the bandwidth is used to obtain a first estimate  of the function to be approximated. Information provided by  this estimate is then used in the bandwidth tuning strategy to  progressively improve the estimation. Bandwidth selection for  nonparametric multivariate regressions is a complex and still  very active field of research [e.g., Vieu, 1991; Wand and Jones,  1994; Herrmann et al., 1995]. We will barely touch it here, since  results obtained with a simple first guess of the bandwidth  already prove to be very satisfactory (see section 6). This first  guess is

h x = C rrxn - 1/5 (25)

where x is either U or SWH, rr x is the standard deviation ofx,  and C is a theoretically determined constant that depends on  the used kernel function (for a Gaussian kernel, C = 1.06).  The theoretical basis for (25) is given, for example, by Simonoff  [1996]. Leaving theory aside, it makes sense to take a band width whose size decreases with the number of measurements  (n) and is proportional to their spreading (rrx).

In this work we will compute series of SSB estimates using  data subsets limited to n = 500 (see next section). The stan dard deviations of U and SWH in our global data set being 3.7  m/s and 1.4 m, relation (25) yields h u • 1 m/s and hswi-i •  0.4 m.

5.2. Determination of the Estimation Uncertainty

Nonparametric estimation theory offers asymptotic results  concerning the uncertainty of the obtained estimates [e.g.,  Hi•rdle, 1990]. In our case, however, this uncertainty can be  evaluated without making use of the rather complex mathe matical arsenal of asymptotic theories. The solution is simply  to estimate the standard deviation of the different SSB esti mates obtained with several data subsets of comparable size.  The natural choice would be to perform a SSB estimation for  each of the 100 repeat cycles of the global data set. This would

8 -

E Oo Oooo• ø

0 0 • 0

0 0 5 J 1• 1 20  u (m/s)

Figure 1. Scatterplot of the (U, significant wave height  (SWH)) measurements in a set of 500 randomly chosen cross over points. The plotted values (squares) correspond to the  ascending arc measurements (x•/). Data come from TOPEX

repeat cycle 46.

require the inversion of a hundred [(n - 1) x (n - 1)]  matrices, where n is typically between 6000 and 7000. This is  feasible but represents a prohibitive amount of computer time  (several days) on the workstation at our disposal. We thus  adopted a purely pragmatic approach and tested the nonpara

metric estimation method with data sets limited to a reason able number of crossover points, 500 in this case. The com puter time needed to invert a [500 x 500] matrix is of the order  of a minute. The whole SSB estimation procedure then takes a  couple of hours. This procedure consists of the following steps'

1. Extract 500 randomly chosen crossover points from each  repeat cycle.

2. Compute ½ for each cycle. Then, using (18), compute  ½(x) over a regular grid of ( U, SWH) values. The used grid size  is 0.25 m/s in U and 0.25 m in SWH.

3. For each grid point, compute the mean and standard  deviation of the individual SSB estimates (½, o-½). The mean  value ½ is our best SSB estimate. Assuming independence of  the individual estimates, the standard deviation of the estima tion error on ½ is

ere = o-•/xf•00 (26)

As will be demonstrated in section 6, the nonparametric SSB  estimates obtained with such a very limited data subset are  already of better quality than those obtained with parametric  models fitted on the global data set.

5.3. Imposing the Constraint

Finally, one has to decide at which ascending arc measure ment one imposes q(xn) = qo and what the value of qo is. The  natural choice would be q(0, 0) = 0, i.e., the SSB must be zero  over a flat sea with no wind. Unfortunately, one never finds a  measurement with strictly zero wave height and wind speed. Of  course, one could impose q(x•) = 0 for the x n measurement

closesto (0, 0), but this is not appropriate for the following  two reasons:

1. The exact location of this Xl• is quite variable from one  data set to another, since the data density close to (0, 0) is low.  Figure 1 shows the typical example of one of our 500-point

15,808 GASPAR AND FLORENS: NONPARAMETRIC ESTIMATION OF SEA STATE BIAS  10

4000

0 , , I , , , , I , , , , I , , , ,

0 5 10 15 2O

u (m/•)

Figure 2. Distribution of altimetric measurements in the (U, SWH) plane. Measurements are binned into  (U, SWH) boxes of width (0.5 m/s, 0.25 m). Isolines of the number of data per box are plotted.

data sets in which the x,, measurement closesto (0, 0) cor responds to U = 1.8 m/s and SWH = 1.6 m. With such a  choice, estimates of q> obtained for individual cycles would be  subjecto a constraint applied at quite variable locations in the  (U, SWH) plane. The mean ½ would thus be of little signifi

cance.

2. Any Xl• close to (0, 0) is far away from the mean of the  (U, SWH) distribution and hence far from most other xzi. The  imposed value of q>(x•) has thus little weight in the estimation  of •. In the worst case the weight can be weak enough to make  the problem numerically undetermined again.

The above discussion actually shows that selecting the value  of x• closesto the mean of the (U, SWH) distribution is a  much better choice: The data density is generally large in the  neighborhood of the mean, and a constraint imposed there has  significant weight in the estimation of most components of •.

In our global data set, the mean of the (U, SWH) distribution  is at (8 m/s, 2.7 m). In all 500-point data sets, there is an x• for  which the distance to this value is less than 0.2 m/s in wind  speed and less than 0.2 m in wave height. Numerical experi ments actually show that the sensitivity of the final SSB esti mate (½) to the choice ofx• is very small as long as x• remains  in a region of high data density (typically inside the 4000-  isoline of Figure 2; see next section).

Having chosen Xll, we must still pick a value for q>o. Ideally,  this value should be such that the final solution meets the  natural requirement q>(0, 0) = 0. This is easily obtained in the  following way: (1) Compute a solution ½ using any reasonable  value of q>o (for example, q>o = -5 cm), and (2) determine  ½(0, 0) and subtract this value from the whole solution. The  so-shifted solution is the final solution. It obviously satisfies  •(0, 0): 0.

6. Results

6.1. Validity Domain of the SSB Estimates

The TOPEX SSB was estimated using the nonparametric  technique implemented exactly as described in section 5. Be fore analyzing the results, it is important to realize that any  statistical estimate of the SSB as a function of (U, SWH) is  meaningful only in the region of the (U, SWH) plane where  altimetric measurements are actually available. The distribu tion of these measurements is shown in Figure 2. This distri bution was computed using both the ascending and descending  arc measurements made at the crossover points in our global  data set. Measurements were binned into (U, SWH) boxes of  width (0.5 m/s, 0.25 m). Isolines of the number of data per box  are plotted. Slightly over 95% of the data fall inside the 400-  isoline. To help the reader visualize this data-rich region, the  boxes containing less than 400 measurements will be system atically shaded in all subsequent figures presenting results in  the (U, SWH) plane. Beware that this number of points con cerns the global data set. The data subset with which SSB  estimation is actually performed contains only a small fraction  (about 8%) of the available measurements. The 400-isoline  thus roughly corresponds to the 30-isoline for this subset.

6.2. Nonparametric Estimate of the TOPEX SSB  and its Uncertainty

Let us first examine the standard deviation (o-½) of the ob tained SSB estimate (Figure 3a). The estimation uncertainty is  minimum in the vicinity of the mean of the data distribution (8  m/s, 2.7 m), that is, in the region of maximum data density. This

minimum is slightly below 1 mm. More generally, o-½ remains  below 4 mm in almost all of the nonshaded region, that is, for

GASPAR AND FLORENS: NONPARAMETRIC ESTIMATION OF SEA STATE BIAS 15,809

12

10

'%,

o. oolo

0.0020

0

0

12

10

2

).010

o

"O.o•o

2O

5 10 15

u (m/•)

'-o./Oo•

-O.o.9o

ø.o.• o

%

0 5 10 15 20  u (m/•)

Figure 3. (a) Uncertainty of the nonparametric estimate ofthe TOPEX sea state bias (SSB) (r%, in meters)  and (b) the corresponding mean estimate (½, in meters).

15,810 GASPAR AND FLORENS: NONPARAMETRIC ESTIMATION OF SEA STATE BIAS

over 95% of the observed (U, SWH) values. This is a surpris ingly good result, given that we only used 8% of the available  data.

The mean estimate (½) of the SSB is shown in Figure 3b. For  comparison, we also plotted (Figures 4a and 4b) the SSB es timates obtained by fitting the BM3 and BM4 models on our  global data set:

BM3(U, SWH) = SWH[0.0019 - 0.0044U + 0.00019U 2]  (27)

BM4(U, SWH) - SWH[-0.021- 0.0035U

remains one legitimate criterion to assess the quality of a SSB  model.

The explained variances for the BM3, BM4, and NP models  are 9.43, 10.04, and 10.53 cm 2, respectively, again, a good result  for the nonparametric solution that has been fitted on only 8%  of the available data. Looking at the meridional distribution of  the explained variance (Figure 5), one observes, like Gaspar et  al. [1994], that BM4 does not explain more variance than BM3  at latitudes smaller than 30 ø but is significantly better at higher  latitudes, where usually above-average wave heights and wind  speeds are observed. The variance explained by NP is system atically larger than that explained by BM3 or BM4. The gain is

+ 0.00014U 2+ 0.0027 SWH]

(28)

modest in low latitudes and more significant at latitudes above  40 ø. This suggests that the accuracy gain provided by NP is

The values of the parameter estimates are very close to those  previously obtained by Gaspar et al. [1994] and Chelton [1994].  Examination of the nonparametric solution (hereinafter de noted NP) shows that the nonparametric method has directly  captured the main known characteristics of the SSB variability  as a function of wind speed and wave height.

1. The estimated SSB is always negative, and its magnitude  is essentially an increasing function of SWH. Variations with  wind speed are less important. This is why the simple BM1  model (equation (13)) is so successful.

2. For a given SWH the magnitude of the SSB increases  with U for wind speeds up to typically 12 m/s and then de creases at higher wind speeds. This is materialized by the  downward and then upward slopes of the SSB isolines when  going from left to right in the nonshaded region. This behavior  was first clearly evidenced by Hevizi et al. [1993] and confirmed  by Arnold et al. [1995]. It motivated use of the BM3 model  (equation (12)) in which the relative bias is a quadratic func tion of U. Still, the comparison of Figures 3b and 4a indicates  that the wind-related variability of the SSB, as captured by NP,  is more complex than simply quadratic.

3. The rate of change of the SSB with SWH is not simply  a constant as hypothesized in the BM1 or BM3 models. It  appears to be relatively large for medium wave heights (SWH  between 2 and 4 m), slightly weaker for small values of SWH,  and markedly weaker for high waves. It even appears to be  close to zero for the highest observed waves (the estimated  SSB is nearly constant in the upper right corner of the non shaded zone). This decrease in the magnitude of the relative  bias with wave height was first detected by Witter and Chelton  [1991a] in Geosat data. It was later observed in TOPEX and  POSEIDON measurements [Gaspar et al., 1994; Chelton,  1994]. The BM4 model (equation (11)) goes some way toward  parameterizing this effect, since it allows the relative bias to  decrease linearly with SWH. Figure 4b shows that the spacing  between the SSB-isolines indeed becomes larger toward high  SWH values but not enough to match the shape of the non parametric solution.

6.3. Skill of the Nonparametric SSB Estimate

The skill of a SSB correction is generally measured in terms  of explained variance. That is the reduction in the variance of  SSH differences obtained when applying the given correction.  As discussed in section 2.2, minimization ofE[SSH 2 - SSH•] 2  is a criterion that must be taken with care in the context of the  calibration of imperfect parametric correction models. Still,  good models will reduce this variance, the absolute minimum  being reached for the exact model. The explained variance thus

particularly important for relatively high values of U and SWH.  The behavior of the mean crossover differences after SSB  correction (often called residuals) is also interesting to analyze.  Figure 6 shows the residuals for the NP model plotted as a  function of the measured wind speed and wave height differ ences (AU, ASWH) at the crossover points. The residuals  binned in A U boxes remain very small, a few millimeters at  most. The residuals sorted according to ASWH are somewhat  larger but still generally below 0.5 cm. For comparison, we also  computed the residuals for the BM3 (equation (27)) and BM4  (equation (28)) models. The results are virtually identical to  those shown by Gaspar et al. [1994]. For both BM3 and BM4  the residuals are largest when binned according to ASWH. For  BM3 they reach 2.5 cm and show marked SWH-related vari ations. They are smaller for BM4 but still exceed 1 cm for  ISWHI -> 4 m. The NP residuals are thus smaller but still  show coherent variations with ASWH. This indicates that there  is scope for further improving the present NP solution. Some  improvements could probably be obtained through a fine tun ing of the bandwidth vector. This (difficult) exercise is avoided  here since there is little point in tuning a solution that makes  use of only a small fraction of the available information. The  first improvement shall certainly come from an improved nu merical implementation of the NP estimation code. It will  allow us to compute NP SSB estimates based on a considerably  larger number of measurements. Bandwidth optimization will  then be warranted.

6.4. Differences Between NP and the Parametric Models

NP, BM3, and BM4 all capture important variations of the  SSB but also exhibit significant differences, plotted in Figure 7.  First of all, these differences clearly have a nonzero mean. NP  is typically 1 cm below BM3 and 2 cm above BM4. These

differences mostly result from the significantly different shapes  of the solutions in the vicinity of (0, 0), a data-poor region.  Discarding these mean differences, it appears that the best  match between NP and the two parametric models is obtained  in the region centered roughly on the mean of the (U, SWH)

distribution, that is, at (8 m/s, 2.7 m). The differences between  NP and BM3 or BM4 exhibit the weakest gradients in this  region. On the contrary, the largest variations in the model  differences are observed in rough surface conditions (U > 10  m/s, SWH > 5 m) or under lighter winds but with SWH values  close to the boundary of the nonshaded zone. The reason is the

following. The NP estimate is designed to fit the observations  over the whole data domain, without any model constraint. On  the contrary, the parametric fits are heavily constrained by the  numerous measurements made close to the mean of the (U,  SWH) distribution. The parametric models are thus tuned to

GASPAR AND FLORENS: NONPARAMETRIC ESTIMATION OF SEA STATE BIAS 15,811

12

10

0

0 5 10 15 20  U (m/s)

12

10

-6h 60

_0.•20

-0.1 10 •

• -0.090

-0.070

0

-0.100

-0.080

0

Figure 4.

5 10 15 20  U (m/s)

Estimates of the TOPEX Ku-band SSB (in meters) obtained by fitting (a) the BM3 and (b) the

BM4 model, on our global crossover data set.

15,812 GASPAR AND FLORENS: NONPARAMETRIC ESTIMATION OF SEA STATE BIAS  the model's parametric form. This led us to develop a new

• 2  E

-1

o NP - BM5

• BM4 - BM3

-60 • -50 • -40 • -30 i -20 i -10 i 0 i 10 i20 i 30 i 40 i 510 6•0  Lotitude (de9rees)

nonparametric formulation of the SSB estimation problem and  to propose a general solution for it, based on a kernel smooth ing technique. This technique was then used obtain the first  fully nonparametric estimate of the TOPEX SSB as a function  of both U and SWH. The obtained solution is based on simple,

conservative choices for the kernel function and its bandwidth  vector.

The basis for the estimation is a global crossover data set  including 100 TOPEX repeat cycles. To reduce the computa tional burden, however, only 8% of the available measure ments were actually utilized. Still, the obtained SSB estimate  (NP) proves to reduce the variance of the crossover differences

more than the estimates obtained by fitting BM3 or BM4  models on the global data set. This is true for the globally  averaged variance and also for the zonally averaged variance.

Figure 5. Meridional distribution of the variance explained  by the BM4 and nonparametric (NP) models minus the vari ance explained by the BM3 model. These variances are com puted using data binned into 10ø-latitude bands.

best match the observations made in this data-rich region.  They are close to the NP estimate in this zone, but away from  it they become more constrained by their specified functional

The largest variance reduction, compared with BM3 or BM4, is  I ' ! ! [ ! !

form than by the few available measurements. The parametric  estimates can thus diverge from the NP solution if the specified  parametric formulation is not totally adequate. This is typically  what we observe in the high-wave/high-wind region. The dif

ference between NP and BM3 varies most quickly, ranging  from-lcmforSWH= 5mupto3cmforSWH= 7m. In  the same range of SWH values the difference between NP and

BM4 exhibits somewhat smaller, but still centimetric, varia tions. These marked differences in the shape of BM3, BM4,

-2

and NP explain why the variances explained by these three SSB  estimates differ most markedly in high-latitude regions, where

o

o

o

o o o

o o o

high waves and high winds are more often observed. The dif -6

ferences between NP and the two parametric models are better  behaved for lower values of U and SWH but can still vary by 1  or 2 cm, especially close to the boundaries of the nonshaded  zone. These differences appear to be significant enough to  allow NP to explain somewhat more variance than BM3 or  BM4 in the calmer low-latitude oceans.

7. Conclusion

A reexamination of the widely used parametric SSB estima tion method showed that parametric models calibrated to min imize the variance of SSH differences do not generally mini mize the variance of the model error: E[SSB - ½]:. In other  words, the so-calibrated models are not true least squares  approximations of the SSB. Simulations show that for com monly used parametric models the difference from the true  least squares solution is typically a few millimeters to a few  -2

centimeters, varying with wave height and wind speed. Such  SSB estimation errors inevitably induce spurious gradients in  the altimeter-derived SSH between ocean regions with differ ent surface conditions. This problem proves to be the conse quence of an imperfect specification of the model's parametric

, , , I , , , I , , , I , , , I , , [ I [ ] [  -4 -2 0 2 4  ASWH (m)

' I ' ' ' [ I ' ' ' ' I ' ' ' ' I ' ' ' ' I

O0 000^000^ . .  00000000• - • •v 0

-10 -5 0 5 10  AU (m/s)

form that affects the model calibration when performed on  measurement differences rather than on the measurements  themselves.

One way out of this problem is to avoid the specification of

Figure 6. Mean residuals for NP as a function of (a) ASWH  and (b) A U. The squares show averages computed on ASWH  bins of width 1 m (Figure 6a) and on A U bins of width 1 m/s  (Figure 6b).

GASPAR AND FLORENS: NONPARAMETRIC ESTIMATION OF SEA STATE BIAS 15,813

12

10

-0.005

_o.O10

NP-BM3

o

o 5 lO 15 20  U (m/s)

12

10

0.020

NP-BM4

o

o 5 lO 15 20  U

Figure 7. Differences (inmeters) between NP and the SSB estimates obtained with (a) BM3 and (b) BM4.

15,814 GASPAR AND FLORENS: NONPARAMETRIC ESTIMATION OF SEA STATE BIAS

obtained at latitudes above 40 ø ,where high-wave/high-wind  conditions are more frequent. It is also in such conditions that  the differences between NP and the parametric models are the  largest. Furthermore, the mean crossover residuals obtained  with NP prove to be significantly smaller than those obtained  with BM3 or BM4. They do not generally exceed 0.5 cm but  still show SWH-related variations. This indicates that the  present NP estimate can still be improved. To this aim, we first  intend to improve the numerical implementation of the  method. This will allow us to obtain nonparametric SSB esti mates based on a larger number of measurements, while keep ing the computation time within reasonable limits. In this way,  we will be able to take full advantage of the information con tained in the enormous, high-quality data set provided by the  TOPEX/POSEIDON mission. Then we shall seek further im provement by fine tuning the bandwidth.

Acknowledgments. The authors thank S6verine Rouzaud, Fran goise Ogor, and Dominique Bouniol, who developed the nonparamet ric estimation software and performed the numerical simulations. The  authors also thank Bernadette Govaerts from the Catholic University  of Louvain (Belgium), who suggested that the two of them collaborate  on the subject of this paper. This work was supported by the Centre  National d'Etudes Spatiales (CNES) under contracts 95/CNES/0436  and 856/CNES/96/0618. It is part of the activities devoted to the prep aration of the forthcoming JASON altimetric mission.

References

Arnold, D. V., W. K. Melville, R. H. Stewart, J. A. Kong, W. C. Keller,  and E. Lamarre, Measurements of electromagnetic bias at Ku and C  bands, J. Geophys. Res., 100, 969-980, 1995.

AVISO, AVISO user handbook: Merged TOPEX-POSEIDON prod ucts, 3rd ed., Rep. AI/I-NT-O2-101-CN, 196 pp., Cent. Natl. d'Etudes  Spatiales, Toulouse, France, 1996.

Barrick, D. E., and B. J. Lipa, Analysis and interpretation of altimeter  sea echo, Adv. Geophys., 27, 61-99, 1985.

Born, G. H., M. A. Richards, and G. W. Rosborough, An empirical  determination of the effects of sea state bias on Seasat altimetry, J.  Geophys. Res., 87, 3221-3226, 1982.

Chelton, D. B., The sea state bias in altimeter estimates of sea level  from collinear analysis of TOPEX data, J. Geophys. Res., 99, 24,995-  25,0{)8, 1994.

Chelton, D. B., E. J. Walsh, and J. L. MacArthur, Pulse compression  and sea level tracking in satellite altimetry, J. Atmos. Oceanic Tech nol., 6, 407-438, 1989.

Douglas, B.C., and R. W. Agreen, The sea state correction for GEOS  3 and Seasat satellite altimeter data, J. Geophys. Res., 88, 1655-1661,  1983.

Fu, L.-L., and R. Glazman, The effect of the degree of wave develop ment on the sea state bias in radar altimetry measurements, J.  Geophys. Res., 96, 829-834, 1991.

Fu, L.-L., E. J. Christensen, C. A. Yamatone, M. Lefebvre, Y. Mdnard,  M. Dorrer, and P. Escudier, TOPEX/POSEIDON mission overview,  J. Geophys. Res., 99, 24,369-24,381, 1994.

View publication stats

Gaspar, P., F. Ogor, P.-Y. Le Traon, and O.-Z. Zanife, Estimating the  sea state bias of the TOPEX and POSEIDON altimeters from cross over differences, J. Geophys. Res., 99, 24,981-24,994, 1994.

Glazman, R. E., A. Greysuckh, and V. Zlotnicki, Evaluating models of  sea state bias in satellite altimetry, J. Geophys. Res., 99, 12,581-  12,591, 1994.

Hiirdle, W., Applied Non-Parametric Regression, Cambridge Univ.  Press, New York, 1990.

Herrmann, E., M.P. Wand, J. Engel, and T. Gasser, A bandwidth  selector for bivariate kernel regression, J. R. Star. Soc., Ser. B, 57,  171-180, 1995.

Hevizi, L. G., E. J. Walsh, R. E. Mcintosh, D. Vandemark, D. E.  Hines, R. N. Swift, and J. F. Scott, Electromagnetic bias in sea  surface range measurements at frequencies of the TOPEX/  POSEIDON satellite, IEEE Trans. Geosci. Remote Sens., 31, 376-  388, 1993.

Melville, W. K., R. H. Stewart, W. C. Koeller, J. A. Kong, D. V.  Arnold, A. T. Jessup, M. R. Loewen, and A.M. Slinn, Measure ments of electromagnetic bias in radar altimetry, J. Geophys. Res.,  96, 4915-4924, 1991.

Parzen, E., On the estimation of a probability density function and the  mode, Ann. Math. Stat., 33, 1065-1076, 1962.

Ray, R. D., and C. J. Koblinsky, On the sea state bias of the Geosat  altimeter, J. Atmos. Oceanic Technol., 8, 397-408, 1991.  Rodriguez, E., and J. M. Martin, Estimation of the electromagnetic  bias from retracked TOPEX data, J. Geophys. Res., 99, 24,971-  24,979, 1994.

Rosenblatt, M., Remarks on some non-parametric estimates of a den sity function, Ann. Math. Stat., 27, 832-837, 1956.

Simonoff, J. S., Smoothing Methods in Statistics, 338 pp., Springer Verlag, New York, 1996.

Vieu, P., Non-parametric regression: Optimal local bandwidth choice,  J. R. Stat. Soc., Ser. B, 53, 453-464, 1991.

Wand, M.P., and M. C. Jones, Multivariate plug-in bandwidth selec tion, Cornput. Stat., 9, 97-116, 1994.

Wand, M.P., and M. C. Jones, Kernel Smoothing, 212 pp., Chapman  and Hall, New York, 1995.

Witter, D. L., and D. B. Chelton, An apparent wave height depen dence in the sea state bias in Geosat altimeter range measurements,  J. Geophys. Res., 96, 8861-8867, 1991a.

Witter, D. L., and D. B. Chelton, A Geosat altimeter wind speed  algorithm and a method for altimeter wind speed algorithm devel opment, J. Geophys. Res., 96, 8853-8860, 1991b.

Yaplee, B. S., A. Shapiro, D. L. Hammond, B. D. Au, and E. A.  Uliana, Nanosecond radar observation of the ocean surface from a  stable platform, IEEE Trans. Geosci. Electron., GE-9, 170-174, 1971.

Zlotnicki, V., L.-L. Fu, and W. Patzert, Seasonal variability in global  sea level observed with Geosat altimetry, J. Geophys. Res., 94,  17,959-17,969, 1989.

J.-P. Florens, GREMAO-IDEI, Universit6 des Sciences Sociales,  Place Anatole France, 31042 Toulouse Cedex, France. (e-mail:  florens@cict.fr)

P. Gaspar, Collect6 Localisation Satellites (CLS), Space Oceanog raphy Division, 31526 Ramonville, France. (e-mail: gaspar@cls.  cnes.fr)

(Received September 11, 1997; revised March 17, 1998;  accepted April 6, 1998.)